’’Model WX-167 - User

’s Manual

Public AlertTM Radio

TABLE OF CONTENTS

IMPORTANT SAFEGUARDS ------------------3

CARE AND MAINTENANCE -------------------4

FCC NOTICE TO USERS ----------------------5

INTRODUCTION ----------------------------------6

MAIN FEATURES ---------------------------------7

LOCATION OF CONTROLS -------------------8-9

CONNECTING POWER-------------------------10

LANGUAGE SELECTION ----------------------11

SETTING THE CLOCK --------------------------11

SETTING ALERT VOLUME --------------------12

SETTING WEATHER CHANNEL -------------12

SETTING SAME LOCATION ------------------13

SET ALERTING OPTIONS ---------------------14

SETTING EVENT BLOCKING -----------------14

BATTERY INSTALLATION ---------------------15

WALL MOUNTING--------------------------------16

BASIC OPERATION -----------------------------16

HANDICAPPED ACCESSORIES -------------17

NOAA MESSAGE MENU -----------------------18

TROUBLESHOOTING ---------------------------19

SPECIFICATIONS --------------------------------20

SERVICE PROCEDURE ------------------------21

WARRANTY ----------------------------------------22

2

Contents in product package

In this package, you will find:

• One Public AlertTM Radio – WX-167

• One AC Power Adapter – 120VAC: 9VAC

• One User’s Manual (this document)

• One Wall Mounting Kit - 1 bracket, 2 screws

2 wall anchors

• One Quick Start Guide

Contact Sima’s Service Center

at 800-345-7462 if you find that any of the

above is missing.

IMPORTANT SAFEGUARDS

When using electrical appliances, the following

basic safety precautions should always be

followed.

CAUTION!

The Manufacturer does not represent this unit to

be waterproof. To reduce the risk of fire, electrical

shock, or damage to the unit, do not expose this

unit to rain or moisture or immerse in water.

CAUTION!

This product has the capability to operate with five

(5) 1.5V AA batteries (either rechargeable Nicad,

NiMH or non-rechargeable alkaline), Please

dispose of batteries properly or recycle at the end

of the life of the battery. Do not incinerate or

compost batteries, as they can explode at high

temperatures. Contact your local authority for

information concerning reclamation and disposal

of rechargeable nickel-cadmium batteries.

Cadmium is a chemical identified by Proposition

65 in the State of California to cause cancer.

CAUTION!

This product has a built-in circuit for charging five

(5) 1.5V AA rechargeable NiCad or NiMHbatteries. When regular alkaline batteries are

used as a backup power source, set the selection

switch in the battery compartment to the

“Alkaline” position to switch off the internal

charger.

CHARGING ALKALINE BATTERIES CAN

CAUSE FIRE AND EXPLOSION AND

ENDANGER LIVES.

DANGER!

To reduce the risk of electric shock, do not

disassemble this product. Take it to a munfacturer

authorized service personnel when service or

repair work is required. Opening or removing

cover voids warranty. This unit does not contain

any parts that can be repaired by the user.

CAUTION!

You should understand the danger of lightning

striking an external antenna and make

appropriate measures to protect yourself from

injury and fire hazard resulted from a lightning

strike on an external antenna.

-When an external antenna is used, make surethat there is sufficient lightning protection on

the antenna and its cable feed. Make sure all

the ground connections are sound and fit. Do

not get close to the unit when it is operating

during a thunderstorm.

-Lower, retract or remove the telescopic antennawhen using an external antenna.

-Contact your external antenna supplier for moreinformation on lightning protection and

grounding issues on your antenna. You could

also get information from lightning protection

device suppliers.

4

Care and Maintenance

-The output jacks for Remote Alert provide a floatingrelay contact output to external warning devices.

The contact will close when an alert broadcast

has been received. Do not exceed the maximum

voltage or current rating of the contacts - 1 Amp

(A) @ 30V DC.

-Wipe unit with a damp cloth occasionally to clean.

Do not use harsh chemicals, cleaning solvents or

strong detergents.

-Do not allow anything to rest on the power cord.

-Do not locate this product where persons walking onit could damage the cord. Keep cord away from

heated surfaces.

-To disconnect, grip the AC adapter and pull it fromthe wall outlet. Never disconnect by pulling on the

cord.

-This product is designed to operate using the ACadapter supplied in a household application or

12V DC for portable use.

- LCD screens can change color in extreme

temperatures. If the unit is exposed to

temperatures below -20ºC (-5ºF) or above +60ºC

(+140ºF), the display may temporarily cease to

function properly, or could result in permanent

damage.

FCC NOTICE TO USERS

The system may cause interference to a TV or radio even when it

is operating properly. To determine whether the system is causing

the interference, turn it off. If the interference goes away, the

system is causing the interference. This equipment has been

tested and found to comply with the limits for a Class B digital

device, pursuant to Part 15 of the FCC Rules. These limits are

designed to provide reasonable protection against harmful

interference in a residential installation.

This equipment generates, uses and can radiate radio frequency

energy and, if not installed and used in accordance with the

instructions, may cause harmful interference to radio

communications. However, there is no guarantee that

interference will not occur in a particular installation. If this

equipment does cause harmful interference to radio or television

reception, which can be determined by turning the equipment off

and on, the user is encouraged to try to correct the interference

by one or more of the following measures:

• Reorient or relocate the receiving antenna.

• Increase the separation between the equipment andreceiver.

• Connect the equipment into an outlet on a circuit differentfrom that to which the receiver is needed.

• Contact your dealer for help.

This device complies with Part 15 of the FCC Rules. Operation is

subject to the following two conditions:

1. This device may not cause harmful interference.

2. This device must accept any interference received, includinginterference that may cause undesired operation.

WARNING: changes or modifications to this unit not

expressly approved by the party responsible for compliance

could void the user’s authority to operate the equipment

IMPORTANT!

Clear voice reception is important, but does not guarantee that

an emergency alert will trigger your unit’s alert siren. To test

actual reception and alert response, your unit must receive a test

or emergency alert signal broadcast. The National Weather

Service (NWS) broadcasts a test alert every week typically on

Wednesday between 11AM and 12PM (noon). The radio will

warn you if it has not received an alert within 10 days.To find out

the specific test schedule in your area, contact your local

National Oceanic and Atmospheric Administration (NOAA) or

National Weather Service (NWS) office. These offices are

usually listed in the telephone book under “US Government”.or

visit the NOAA website athttp://www.nws.noaa.gov/nwr/

REMEMBER – FOR YOUR SYSTEM TO BE EFFECTIVE, YOU

MUST PLACE IT WHERE IT CAN RECEIVE AN EMERGENCY

ALERT SIGNAL AND WHERE YOU CAN HEAR ITS ALERT

SIREN OR SEE ITS FLASHING LIGHTS.

INTRODUCTION

Congratulations on your purchase of the First

Alert® brand Public AlertTM Radio, Model WX-167.

This product complies with the exacting standards for

Public AlertTM devices as set by CEA -2009, and will

provide reliability, long life and outstanding

performance. The WX-167 receives both audio and

digitally encoded civil and weather emergency alert

messages transmitted from NOAA broadcast towers.

NOAA has more than 900 towers in the 50 states and

near adjacent coastal waters, Puerto Rico, the U.S.

Virgin Islands and the U.S. Pacific Territories.

The WX-167 Public AlertTM Radio receives all 7

NOAA weather channels and uses Specific Area

Message Encoding (S.A.M.E.) technology to identify

specific areas, like a county or portion of a state.

To simplify programming, the WX-167 has a builtin

database of over 3000 state and county locations.

You can select up to 20 locations for your radio to

respond to.

NOTE: You must select the correct weather

channel to receive warnings for the state/county

codes that you have programmed.

6

How it works

When NOAA broadcasts a message, the WX-167

decodes the message and displays it on the LCD

screen. One of three color-coded LED lights will light

up identifying the message as an “Advisory” (yellow),

“Watch” (orange) or “Warning” (red). External

handicapped accessory devices are triggered, if

connected. The alert siren sounds and the radio

automatically switches to the voice broadcast.

There are optional settings which you can use,

such as Event Blocking and Alerting Options, to

customize the response of your radio to an alert.

Working with other federal agencies and the FCC’s

Emergency Alert System (EAS), NOAA is the single

source for weather and civil emergency information

available to the public. For up-to-date information on

NOAA and weather frequencies used in your area,

check the NOAA web site at

http://www.nws.noaa.gov/nwr.

Your can also call 1-888-NWR-SAME

(1-888-697-7263) for further information.

The main features of the WX-167 Public Alert Radio include:

Alert Function

Your Public AlertTM Radio will automatically sound an

alert tone siren and flash the corresponding LED

color indicator when it receives a weather or civil

emergency broadcast from NOAA.

Specific Area Message Encoding (S.A.M.E.)

Allows you to store up to 20 different location codes into

memory. Each code identifies a specific geographic

area typically a county (defined by NWS). Setting the

S.A.M.E. helps reduce unnecessary alerts.Your WX-167will sound an alert when a civil or weather emergency is

declared in your area.

Alphanumeric Liquid Crystal Display (LCD)

The LCD displays the NOAA weather channel number,

time, programming information and alert/status

description. The LCD has a unique tilt view function to

provide optimal visibility.

Display Backlight

Allows you to read the LCD display in low light

situations. When powered from the AC adapter or 12V

DC source, the backlight will stay on continuously. To

conserve power from the backup batteries during a

power failure, the backlight will stay on for about 10

seconds when you press a key.

Non-Volatile Memory

Channel and location settings are stored in non-volatile

memory, even when AC and battery power is removed.

Remote Alert

This feature can be used to control external handicapped

accessible devices such as strobe lights, pillow vibration

modules and external sirens.

Alert Audio Output

Allows connection of the siren and audio signal to a

Public Address system, amplifier, un-amplified external

speaker or two-way radio for special alert applications.

The volume contol knob controls the audio output. The

radio speaker is disabled when using this feature.

External Antenna (F connector)

Allows connection to an optional external antenna (not

included) to improve its reception in fringe or low

reception areas such as a shelter.

Rear Power Jack:

Connects to 9V AC wall adapter or 12VDC car cigarette

plug adapter.

Battery Backup

During AC power blackout periods, the unit will maintain

normal operation on five AA backup batteries.

Rechargeable Ni-Cad or NiMH batteries as well as

alkaline batteries may be used. A switch located in the

battery compartment is provided to select the type of

battery used. Typically, alkaline batteries will provide up

to 24 hrs of operation while rechargeable NiCad and

NiMH batteries will provide up to 12 hours. See battery

installation section (page 15) for more information.

LOCATION OF CONTROLS AND FUNCTIONS

WX-167

1 23 45

6 7 8 9 10

11

12

17 18

20

19

1514

13 163LCD 2

2322

1

Screen

21

4

1. IntegratedTelescopic Antenna

2. Remote Alert Jack (3.5 mm)

3. Alert Audio Out Jack (6.2 mm)

4. External Antenna Jack (F Connector)

5. DC Power Jack

6. Liquid Crystal Display (LCD)

7. Warning Indicator (red) LED

8. Watch Indicator (orange) LED

9. AdvisoryIndicator (yellow) LED

10. Power Indicator (green) LED

11. Function Knob

12. ALERT Volume/OFF Switch

13. Volume Knob

14. Speaker

15. PREV Button (Denoted as “<”)

16. SAVE Button

17. PROGRAM Button

18. NEXT Button (Denoted as “>”)

19. Radio OFF Button

20. Radio ON Button

21. Battery type switch

22. PC programming switch

23. PC connection jack

LCD Screen

1. Time of Day with AM/PM indicator

2. Alert siren setting

3. Number of stored alerts that are active

4. Scrolling text for messages and programmenus, date.

8

Description of controls and functions

1. Integrated Telescopic Antenna – Fully extend the antenna

and orient it for best reception.

2. Remote Alert (3.5 mm Jack) – provides contact closure

to activate external devices.

3. Alert Audio Out (6.2 mm Jack) – Mono jack allows

connection to a Public Address system or two-way radios

for special applications.

4. External Antenna (F Connector) – F connector allows

you to connect to optional external antenna (not included).

5. DC Power Jack – Connect AC Adapter or 12V DC from car

cigarette adapter (not included).

6. LCD Display – Displays time and alert messages. Also

used to display different options when programming.

7. “Warning” Indicator (Red) – Indicating that a “Warning”

is in effect. See alert table on pg. 18.

8. “Watch” Indicator (Orange) – Indicating that a “Watch” is

in effect.See alert table on pg. 18.

9. “Advisory ” Indicator (Yellow) – Indicating that an

“Advisory” is in effect. See alert table on pg. 18.

10. Power Indicator (Green) – Indicates the unit has power.

11. Function (Scroll) Knob – Allows you to scroll “Up” and

“Down” through option selections when the radio is in

programming mode.

12. Alert Volume Off/Hi/Low Switch: Setting the switch to

the OFF position will mute the alert siren. The radio will

still receive alerts but will not sound the siren. Use the event

blocking feature to reduce unwanted alerts.

13. Volume Control – Controls listening volume and remote

output volume level. This control will not affect the volume

level of the alert siren.

14. Speaker

15. Prev Button – Use to scroll back through menu options

while programming.

16. Save Button – Allows you to confirm the menu selections

made while programming and stores the selections into

the unit’s internal memory.

17. Program Button – Press and hold for 3 seconds to put

the radio into programming mode. Press once again to

revert without saving to normal operation.

18. Next Button – Use to scroll forward through menu options

while programming.

19. Radio Off Button – When the radio receives an alert, it

activates the alert siren. Press this button to stop the alert

siren and revert to standby mode. Also press to mute voice

braodcast.

20. Radio On Button – Allows you to un-mute the speaker

and listen to the voice broadcast.

20. Battery Compartment – Open the cover to install or

replace the batteries.

21. Battery Selection Switch – Select “Alkaline” or “NiCad”

according to back-up battery type. Set switch to the right

for alkaline and to the left for rechargeable. See battery

installation instructions on page 15 for diagram.

22. PC programming switch 23.

PC Connection jack

Setting up your Public AlertTM radio

1. Choosing a place for your radio

Your WX-167 is designed for desktop or wall mount

installation. Place the unit where it can receive an

emergency alert signal broadcast and where you can

hear its alert tone or see the message indicator lights.

The best location to install the WX-167 is:

-Clear of any obstructions.

-Near windows or large openings for bestreception.

-Near an AC Power outlet

-Near to where the user(s) would normally stay,

such as a bedroom, living room or office.

• Keep the receiver away from interference sources

e.g. motors, computers, TV or microwave ovens.

• All LCDs have a preferred viewing angle when the

display contrast is at maximum. Adjust LCD flip door

for best viewing angle.

2. Connecting the Power

USING A STANDARD 120V AC OUTLET

1. Plug the AC adapter cord into the 9V AC input jackon the back of the unit.

2. Plug the adapter into the wall outlet.

Note: The internal battery charger will automatically

begin to charge the batteries if the “Rechargeable”

setting is selected. Keep the unit plugged in at all

times. The unit will be ready to use in the event of a

power failure.

VEHICLE USE (12V DC)

Use an optional 12V DC car adapter (not included).

Plug the adapter cord into the 12V DC input jack on

the back of the unit. Plug other end into vehicle’s

cigarette lighter or accessory outlet.

10

Note: The radio will beep when choosing

programming options. To adjust the volume of the

beeping, set the Alert Volume switch to the desired

position. Reset the Alert Volume switch when done.

3. Set-up and Language Selection

When the radio is powered up, the LCD screen will

illuminate and the four lights located above it will light

up for 3 seconds and it will beep 3 times.

Note: The radio will prompt you to select a language

the first time you program it. You will not have to do so

when programming the radio at other times.

1. When powered up, the radio will prompt you to

HOLD PROGRAM TO SETUP RADIO in English

then in Spanish. Press and hold the PROGRAM

key for 3 seconds.

2. The LCD screen will display ENGLISH. Use the

FUNCTION KNOB to toggle between English or

Spanish language. Select your language choice

and press the NEXT ( “>”) key to save your

selection. After setting the language selection

proceed to Step 2 in the Clock/Calendar menu.

Note: To reset language selection, disconnect power

from the unit, including batteries. Re-connect power

while pressing the PROGRAM button. Repeat steps 1

and 2 above.

4. Setting the Clock/Calendar

1. When powering up the radio, you will be prompted

to HOLD PROGRAM TO SETUP RADIO. Press

and hold the PROGRAM key as prompted.

2. The SET DATE/TIME menu will be displayed on

the LCD screen. Press the NEXT ( “>”) key to

enter the SET DATE/TIME menu.

3. SET YEAR will be displayed on the screen. Turn

the FUNCTION knob to select the current year.

4. Press the NEXT ( “>”) key and SET MONTH will be

displayed on the screen. Turn the FUNCTION

knob to select the desired month.

5. Press the NEXT ( “>”) key and SET DAY will be

displayed on the screen. Turn the FUNCTION

knob to select the correct day of the month.

6. Press the NEXT ( “>”) key to enter the TIME ZONE

menu. Turn the FUNCTION KNOB to set the Time

Zone -as follows: Eastern - 0500, Central 0600,

Mountain - 0700 & Pacific - 0800.

7. Press the NEXT ( “>”) key and DAY SAVING will

be displayed on the screen. Turn the FUNCTION

knob to select either Daylight Savings ON or OFF.

Set to ON if it is currently Daylight Savings time in

your location.

8. Press the NEXT ( “>”) key and CLOCK will be

displayed next. Turn the FUNCTION knob to select

between the 12-hour or 24-hour clock systems.

9. Press the NEXT ( “>”) key and SET TIME will be

displayed on the screen. Use the FUNCTION knob

to scroll to the current time.

10. Press the SAVE key to save all of the settings for

your clock/calendar and return to normal operation.

5. Set Alert Volume

Make sure the alert siren volume switch, located on

the left hand side of the radio, is turned is set to the

volume level (Lo/Hi) to your preference.

Note: If the alert volume switch is in the OFF

position, the radio will not sound an alert tone.

6. Selecting the Weather Band Channel

To receive alerts for your area, you must select

the correct weather channel. NOAA broadcasts on

seven channels, in 25KHz increments, between the

frequencies of 162.400MHz and 162.550MHz. To

identify the appropriate channel for your area, visit the

NOAA web site at:

http://www.nws.noaa.gov/nwr/indexnw.htm

12

The table on the NOAA website includes the

correct weather band channel for your area, the

SAME code and the location of the broadcast tower.

Look for the state in which you live and then search

the list for your county. Use the channel listed for your

county when programming the radio.

If you are unable to access the NOAA website, you

can select a weather channel by listening to each

channel and choosing the one with the best reception.

In most cases this will be the appropriate channel,

however, sometimes broadcast areas overlap. The

best reception may not originate from the tower broadcasting

to your area.

If you are unable or not sure how to select the

correct weather band channel, please call Sima

customer service at 800-345-7462 for assistance.

Selecting the WB channel for your listening area:

1. Press the radio ON button to hear the voicebroadcast. Adjust the volume knob as needed.

2. Press and hold the PROG key until SET DATE/

TIME is displayed on the screen.

3. Turn the FUNCTION knob until SET WB

CHANNEL is displayed on the screen.

4. Press the NEXT ( “>”) key to enter the WB

Channels menu.

5. Turn the FUNCTION knob to select the correctchannel for your area, as identified by NOAA.

Adjust antenna to ensure clear reception.

6. Press the SAVE key to save the channel and return

to normal operation.

7. Setting the Specific Area MessageEncoding (S.A.M.E.) location

The WX-167 features a built-in database of the

3,000 locations identified by NOAA in the United

States. Your Public AlertTM radio is programmed at

the factory to receive alerts for the entire county. This

may include alerts that do not affect you. Selecting

your S.A.M.E. location can reduce unnecessary alerts.

To select your S.A.M.E. location, simply select your

state, county and county subdivision at the menu

prompts as described below.

To set your SAME location:

1. Press and hold the PROG key until SET DATE/

TIME is displayed on the screen.

2. Turn the FUNCTION knob to select the SETUP

LOCATIONS menu. Press the NEXT ( “>”) key to

enter the SETUP LOCATIONS menu.

3. SET LOCATION 01 will be displayed on the

screen. Press the NEXT ( “>”) key to enter the

SELECT STATE menu.

4. Turn the FUNCTION knob until you locate the

desired state. Press the NEXT( “>”) key to enter

the SELECT COUNTY menu.

5. Turn the FUNCTION knob until you locate the

desired county. Press the NEXT ( “>”) key to enter

the SELECT SUBDIVISION menu.

6. Turn the FUNCTION knob to scroll through the

subdivisions within a county. For ex. NorthEast

Note: Setting the subdivision to ANY (0) will

receive all alerts broadcast in the entire

county.

7. Press the NEXT ( “>”) key when you have selected

the subdivision of your county .

8. Press the SAVE key to save your SAME location

and return to normal operation.

NOTE: Because broadcast areas overlap you may

want to set more than one S.A.M.E. location. To set

more than one S.A.M.E. location, you must complete

the above steps for each location desired. Repeat

steps 1 and 2 above. Upon entering the SETUP

LOCATIONS menu in step 3, turn the FUNCTION

KNOB to select an alternate location, press the NEXT

( “>”) key and proceed as prompted through step 8.

8. Selecting Between Alerting Options

The WX-167 offers four different alerting options for you to

select from. Each choice will make the WX-167 respond

differently when an alert is received. Your radio is set at

the factory to Option 1 - Voice/Message.

The four choices are:

1. VOICE/MESSAGE: The radio will sound the alert siren

for 5 seconds then switch to the voice broadcast for the

duration of the message. (About 15 to 60 sec.)

2. SIREN/MESSAGE: The radio will sound the alert siren

for the duration of the message. (About 15 to 60 sec.)

3. VOICE/EVENT: The radio will sound the alert siren for

15 seconds and then switch to the voice broadcast for

the duration of the alert. (About 15 min to 6 hrs.)

4. SIREN/EVENT: The radio will sound the alert siren for

the duration of the alert.(About 15 min to 6 hrs.)

To select an alert mode:

1. Press and hold the PROG key until SET DATE/TIME is

displayed on the screen.

2. Turn the FUNCTION knob until ALERT OPTIONS is

displayed on the screen. Press the NEXT ( “>”) key to

enter the ALERT OPTIONS menu.

3. Turn the FUNCTION KNOB to select the alert type you

want .

4. Press the SAVE key to save the option you selected.

14

8. Event Blocking

To reduce unwanted alerts, such as weekly tests, the WX167

features the ability to block specific alerts. The WX-167

is programmed at the factory with some events blocked.

See the alert list on page 18 for more information.

To set event blocking options :

1. Press and hold the PROG key until SET DATE/TIME is

displayed on the screen.

2. Turn the FUNCTION knob until SETUP BLOCKING is

displayed on the screen. Press the NEXT ( “>”) key to

enter the SETUP BLOCKING menu.

3. The screen will display a 3-letter message code followed

by either the words “BLOCKED” or “NOT BLOCKED”.

For example: ADR-BLOCKED

The full text name of the message will scroll

across the screen.

For example: ...Administrative Message..

4. Use the FUNCTION KNOB to scroll through the list of

events. Press the NEXT ( “>”) key to select the event

you want.

5. Use the FUNCTION KNOB to toggle between the

BLOCKED and NOT BLOCKED options. Select the

blocking option of your choice.

6. Press the SAVE button to set your blocking options.

Repeat steps 4 - 6 above to block other alerts.

7. Press the PROGRAM key to return to the main menu.

Note: Blocking an event disarms the siren and remote

output only. The LED light indicators and text message

will still appear when the event is received.

9. Battery Installation

The WX-167 uses 5 AA batteries for back-up power.

It can run on non-rechargeable alkaline, or

rechargeable Nickel Cadmium (Ni-Cad) or Nickel

Metal Hydride (NiMH) batteries. Use the slide switch

located inside the battery compartment (Fig. 1) to

select the battery type. To select “Alkaline” when

alkaline batteries are installed, push the switch to the

right. To select “Ni-Cad” when using rechargeable

batteries, push the switch to the left. (See Fig. 1). It is

recommended that rechargeable batteries be

replaced every three years.

DO NOT CHARGE ALKALINE BATTERIES!

Always set the battery select switch to “Alkaline”

before installing alkaline batteries. Using the

“NiCad” setting with alkaline batteries may cause

a fire hazard.

To install or replace the batteries:

-Turn the unit upside down. Locate the batterycompartment door tab (Fig. 2), press down on the

door tab and lift cover up to open

-Insert the batteries into the battery compartment. Besure to follow the polarity markings. (Fig. 3)

-Place the batteries in the battery compartment andreplace the cover.

NOTES: Rechargeable batteries must be charged for

at least 24 hours before they are ready. Rechargeable

batteries provide approximately 8-12 hours of

operation time. Alkaline batteries will provide up to 24

hours of operation time. To maintain maximum backup

time, do not disconnect AC power supply from the unit

while in use. Remove the batteries when the unit is

not used for a prolonged period of time.

15

Fig. 1

Fig. 3

Fig. 2

NiCad Alkaline NiCad Alkaline

10. Wall Mount Installation

Note: Be sure to install batteries before wall mounting

1. Remove the wall mount bracket from the package.

Hold the bracket at the mounting location with the

narrow edge at the top. Use it as a template to

mark the positions for the screw holes on the wall

or other mounting

surfaces.

2. Drill the two screw holesand fix the screws onto

the wall, using the

drywall anchors if

necessary, Leave a

space of about 3mm (1/

8”) between the head of

the screw and the wall.

3. Put the radio and thewall mount bracket

together with the AC adapter plug installed. Make

sure all the clips are properly set and the radio is

secured.

4. Hang the radio with the wall mount onto the twoscrews on the wall, make sure the installation is

secure. Connect the AC adapter to a power outlet

Basic Operation

After programming your WX-167, you are ready to use

it . Place the radio where it gets the best reception. To

listen to the weather channel press the Radio ON

button. To Turn off the weather channel press the

Radio OFF button. To turn the siren or voice off when

an alert is received, press the OFF button.

To Review Alert Messages

The WX-167 will store up to 10 currently active alert

messages in its memory. Alerts are removed automatically

when the time for the event expires. Upon receiving a new

alert, the oldest inactive alert will be deleted. If all stored

alerts are active, it will delete the oldest active alert. The

number of active alerts is displayed on the LCD screen in

the upper right corner. Press the Next “>” or Previous “<”

button to play the alert denoted by the circled number.

TEST SIGNAL FROM NOAA:

NOAA sends out test signals weekly for users to verify that

your Public Alert radio is working properly. Your radio is

programmed at the factory to block the alert siren for the

weekly test signal. When the weekly test signal is received:

1. The LCD screen will display the text message

“The National Weather Service has issued a required

weekly or monthly test for your county until ####”

2. The radio will beep for 15 sec. The siren will not sound.

3. Weekly and monthly tests are stored in Alert Memory.

Notes and Tips - Appendix A

Setting Daylight Saving - This setting allows the radio to

convert UTC time used by NOAA to your local time of day.

It does not automatically set your clock to adjust for daylight

savings time. Be sure to set the daylight savings time to ON

and OFF in accordance with time changes in your area.

Note: The WX-167 will automatically notify you if it has not

received a test message in the previous 10 days, by lighting

all four LED indicators and displaying “10 DAY ALERT

SENTRY” on the LCD screen.

CAUTION!

Storm front activity can continue even though the

alert’s effective time has expired. The end of an alert

does not mean that the related emergency is over.

Note: Time keeping functions will be lost and the clock will

not reset to the correct time if AC power is disconnected

from the unit and there is no battery back-up power. To

reset the clock upon restoring power, follow the Clock/

Calendar setting instuctions on pages 11-12.

Note: For best reception, lift up the built-in telescopic

antenna and fully extend it vertically. The channel that gives

you the best reception may not be the one that broadcasts

weather information for your own location or county. If you

can’t get good reception either move the radio or use an

external antenna

Handicapped Accessories - Appendix B

The WX-167 Public Alert radio is equipped to

connect a variety of special needs accessory devices

including strob lights, pillow vibration modules and

remote sirens.

Once connected, these devices will automatically

be activated when the radio receives an advisory,

watch or warning for the duration of the alert or until

you press the Radio OFF button.

To connect a special needs accessory, plug the

device into the Remote Alert jack on the rear panel of

the radio shown in Fig. 1. This jack requires a 3.5 mm

connector.

Fig. 1

Note: If an alert is blocked, the accessory will not

be activated. The Alert Volume switch does not affect

devices connected to the remote alert ouput.

To purchase special needs accessories contact

Sima Products at 800-345-7462

Alert Description Table: The table below lists the alerts broadcast by NOAA. At the factory, your radio has

been programmed with some alerts Blocked (B) as noted below. In addition, some NOAA alerts are

Unblockable (UB) as noted below. To block additional events, see the Event Blocking instructions on page 14.

(B) Administrative message (UB) Earthquake Warning

Avalanche Watch (UB) Immediate Evacuation

Avalanche Warning Evacuation Watch

(UB) Biological Hazard Warning (UB) Food Contamination Warning

Boil Water Warning Flash Flood Watch

Blizzard Warning Flash Flood Advisory

Child Abduction Emergency Flash Flood Warning

(UB) Civil Danger Warning Flood Watch

(UB) Civil Emergency Message Flood Advisory

Coastal Flood Watch Flood Warning

Coastal Flood Warning Fire Warning

(UB) Chemical Hazard Warning Flash Freeze Warning

(UB) Contaminated Water Warning Freeze Warning

(UB) Contagious Disease Warning (UB) Hazardous Materials Warning

Dam Watch Hurricane Advisory

(UB) Dam Break Warning Hurricane Watch

Dust Storm Warning (UB) Hurricane Warning

(UB) Emergency Action Notification High Wind Watch

(UB) Emergency Action Termination High Wind Warning

Iceberg Warning

(UB) Industrial Fire Warning

(UB) Local Area Emergency Tornado Watch

(UB) Law Enforcement Warning 911 Telephone Outage

(UB) Land Slide Warning (UB) Tornado Warning

(B) National Audible Test (Tropical Storm Watch

(B) National Information Center (UB) Tropical Storm Warning

(B) Network Notification Message (UB) Tsunami Watch

(B) National Periodic Test (UB) Tsunami Warning

(B) National Silent Test (B) Transmitter Backup On

(UB) Nuclear Power Plant Warning (B) Transmitter Carrieer On

Power Outage Advisory (B) Transmitter Carrier Off

(B) Practice/Demo (B) Transmitter Primary On

(UB) Radiological Hazard Warning (UB) Volcano Warning

(B) Required Monthly Test Wild Fire Watch

(B) Required Weekly Test (UB) Wild Fire Warning

Special Marine Warning Winter Storm Watch

Special Weather Advisory Winter Storm Warning

(UB) Shelter In-Place Warning Unrecognized Watch

Severe Thunderstorm Watch Unrecognized Advisory

Severe Thunderstorm Warning (UB) Unrecognized Warning

Severe Weather Advisory

Trouble Shooting

Problem

Possible Cause and Solution

POWER light

AC power may not be connected.

does not come

Make sure AC adaper is properly

on

plugged into the radio and the wall

outlet

POWER light is

AC power has been removed. Unit is

blinking

running on backup battery power.

Check AC adapter connections to

restore power.

Text is in the

Language selection is incorrect. To

wrong language

reset language selection, follow

instructions on page 11

Siren does not

1. Alert Volume switch may be in OFF

sound

position. Set switch to Lo or Hi

position.

2. Alert Audio Output jack may be in

use. Disconnect external device

from Alert Audio Output jack.

3. Alert is blocked. - To Unblock alert

see Event Blocking instructions on

page 14.

Problem

All 4 LED

indicator lights

are on and

“10 Day ALert

Sentry” is

displayed on the

LCD screen.

No sound from

the radio’s

speaker

There is only

static when I

press the radio

ON button.

Possible Cause and Solution

Your radio has not received an alert

for 10 days or more. Weather channel

or S.A.M.E. location programming

may be incorrect.

Re-program weather channel and

S.A.M.E. according to instructions onpages 12-13. Also make sure radio is

getting clear reception.

Check to make sure the Volume Knob

is not at the lowest level.

Also check the Alert Audio output.

When a device is connected to the

Alert Audio output, the radio’s speaker

is muted.

1. May have programmed incorrect

weather channel. To reset weather

channel, follow instructions on

page 12.

2. Poor reception. Move radio location

or use external antenna.

19

WX-167 Specifications

FREQUENCY COVERAGE – WEATHER CHANNELS

Channel 1............................... 162.400 MHz

Channel 2............................... 162.425 MHz

Channel 3............................... 162.450 MHz

Channel 4............................... 162.475 MHz

Channel 5............................... 162.500 MHz

Channel 6............................... 162.525 MHz

Channel 7............................... 162.550 MHz

SPECIFICATIONS

Power Requirements:

9V AC or 12V DC 200mA (120V: 9V - AC power adapter)

5 AA Alkaline, Ni-Cad or NImH Batteries (not included)

Integrated Antenna: 21” Telescopic Rod Antenna

External Antenna (not included):50 Ohms

External Antenna Connector: F type jack

Remote Alert Connector: 3.5mm mono audio jack

Remote Alert Output Rating: 1.0A@30VDC

Audio Output Power: 200mW (max)

Size: 5-1/2”(L) X 8-1/4” (W) X 1-3/4” (D)

Weight: 13.5 oz. (without batteries)

ACCESSORIES

-AC Power Adapter (Included)

-Mounting Bracket (Included)

-Car Power Adapter (Optional – Not Included)

-External Antenna (Not included)

20- Handicapped accessories (not included)

SERVICE PROCEDURE

To send the radio in for service, call 800-345-7462.

You may be asked to send your unit in for service.

It will be necessary to furnish the following, in

order to have the product serviced and returned.

1. For Warranty Repair, include some form of proof-of-

purchase.

2. You must include the Public Alert System Receiverunit and power adapter.

3. Enclose a description of the problem with the unit.

Include a typed or clearly printed name, address,

and phone number where you can be contacted.

4. Pack the unit securely to prevent damage in transit.

If possible, use the original packing material.

5. Ship the unit prepaid and insured by way of atraceable carrier (to avoid loss in transit) such as

United Parcel Service (UPS), Roadway Parcel

Service (RPS) or First Class Insured

Mail to:

Sima Products Corporation

Attn: Customer Service

140 Pennsylvania Ave, Bldg #5

Oakmont, PA 15139

The Manufacturer is not responsible for units not

received if the package has not been properly

insured.

6. If the unit is in warranty, upon receipt of your unit itwill either be repaired or exchanged.

Please allow approximately 2 to 4 weeks for your

radio to be serviced. If the unit is out of warranty, we

will notify you of the repair charge or replacement

charge. If you have any questions, please call 800-

345-7462 for assistance.

21

90-Day Limited Warranty

Sima Products Corp. (“Company”) warrants that if the

accompanying product proves to be defective to the

original purchaser in material or workmanship within

90 days from the original retail purchase, the

Company will, at the Company’s option, either repair

or replace same without charge (but no cash refund

will be made).

Conditions

1. You must deliver, mail or ship the product, togetherwith the original bill of sale and this limited

warranty statement as proof of warranty coverage

to:

Sima Products Corporation

Attn: Customer Service

140 Pennsylvania Ave, Bldg #5

Oakmont, PA 15139

It is recommended that you call Sima at the number(s)

listed below to obtain a return authorization number:

412-828-3700 or 800-345-7462

or email: firstalert@ simacorp.com

2. This warranty is void if any defects are caused byabuse, misuse, negligence or unauthorized

repairs.

3. All liability for incidental or consequential damagesis specifically excluded. Some states do not allow

the exclusion or limitation of incidental or

consequential damages, so the above limitation or

exclusion may not apply to you. This warranty

gives you specific legal rights, and you may also

have other rights which vary from state to state.

Limitation of Liability and Remedies

Sima shall have no liability for any damages due to

loss of profits, loss of use or anticipated benefits,

or other incidental, consequential, special or

punitive damages arising from the use of, or the

inability to use, this product, whether arising out of

contract, negligence, tort or under any warranty,

even if Sima has been advised of the possibility of

such damages. Sima’s liability for damages in no

event shall exceed the amount paid for this

product. Sima neither assumes nor authorizes

anyone to assume for it or any other liabilities.

Notes

23

© 1999, 2002, 2004 First Alert®: all rights reserved.

First Alert® is a registered trademark of the First Alert Trust, used under license.

PN21703

Printed in China